CLASS 31422R5

1. This piping class is derived from SP-2163 piping class 31422R2 rev.04.

2. Corrosion allowance changed to 0 mm.

3. Assemblies have been updated based on memorandum UEMS_2022_01_REV.02 released on 13 Oct. 2022.

Design limits

| Temperature (°C) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | -29 | 0 | 50 | 60 |
| Pressure (Barg) |  |  |  |  |
| DN 50- 600 | 50.5 | 50.5 | 50.1 | 49.38 |

| Notes |
| --- |
| - Design limits acc. To ASME B16.5 flange rating mat. grp 1.1 and SP-2094. - For Crude oil & Multiphase services, design temperature is to be restricted to 60° C - This piping class covers only for oil service. - For basis of design see DEP 31.38.01.10 – Gen - Use of coated bolt sets should be considered when exposed to marine environments. - Piping class design meets full vacuum at ambient condition. - Due to supply chain constraints the wall thickness for pipe DN300 (NPS12) SCH20 (6.35 mm) is ordered as per MESC description with wall thickness 7.14mm. (Applicable for 11422 series only) - Minimum branch size is restricted to DN 50, due to limitation in PE/ rotoining - Threaded branch fitting is applicable only for Annulus vent connection on PE lined spools. - Pre – fabricated carbon steel spool shall have flanged end and shall be limited to L, T & Z shapes as given in this piping class. However, exact size of the spool shall confirm to vendor’s recommendation for Rotolining. - Gaskets are not required for the PE / Roto lined flanged connections. However, Retaining rings shall be provided around PE / Roto lined flanged connections in order to limit the compression of PE / Roto lined flanges while bolting. Width of the retaining ring shall be calculated considering 20% compressibility for rotolining and 15% for PE liner. - DN 50 Monoflange slimline valves shall be selected for clean services only. - In the assemblies, Monoflange valve with 10mm bore fitted with 3/8" compression tube fitting is listed based on PDO Instrument Hook-up diagram. - The use of red. bore ball valves is considered most economical. Use full bore ball valves only when necessary for process/operating requirements - The use of triple eccentric butterfly valves and dual plate check valves is considered most economical. - Use gate valves and swing check valves only where fouling and abrasion are negatively affecting their functionality - HDPE / Rotolining shall be in accordance with SP-2094 and DEP 31.40.30.34-Gen - PE liner thickness provided in this piping class is preliminary considering 3 bar collapsible pressure for Looser Liner type. However, vendor shall confirm / calculate wall thickness for PE Liner as per SP-2094 for the applicable liner type. - Thickness of the stub end at flanged ends shall be twice the thickness of the liner |  |

Table of schedules

| DN | Schedule |
| --- | --- |
| 15 | 80 |
| 20 |  |
| 25 |  |
| 40 |  |
| 50 | 40 |
| 80 | 40 |
| 100 | 40 |
| 150 | 40 |
| 200 | 30 |
| 250 | 30 |
| 300 | 30 |
| 350 | 30 |
| 400 | 30 |
| 450 | 30 |
| 500 | 30 |
| 600 | 30 |

Branch connections 90 degrees

| Branch Size |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Run size | 50 | 80 | 100 | 150 | 200 | 250 | 300 | 350 | 400 | 450 | 500 | 600 |
| 600 | E | E | E | E | E | E | B | B | B | B | B | A |
| 500 | E | E | E | E | E | E | B | B | B | B | A |  |
| 450 | E | E | E | E | E | B | B | B | B | A |  |  |
| 400 | E | E | E | E | E | B | B | B | A |  |  |  |
| 350 | E | E | E | E | B | B | B | A |  |  |  |  |
| 300 | E | E | E | B | B | B | A |  |  |  |  |  |
| 250 | E | E | B | B | B | A |  |  |  |  |  |  |
| 200 | E | E | B | B | A |  |  |  |  |  |  |  |
| 150 | E | B | B | A |  |  |  |  |  |  |  |  |
| 100 | B | B | A |  |  |  |  |  |  |  |  |  |
| 80 | B | A |  |  |  |  |  |  |  |  |  |  |
| 50 | A |  |  |  |  |  |  |  |  |  |  |  |

| Code | Explanation of characters |
| --- | --- |
| A | Equal tee |
| B | Reducing tee |
| E | Branch outlet |

| Carbon steel Sour +PE/ Roto lining |  | Plant | On Shore |
| --- | --- | --- | --- |
| Class no. | 31422R5 | Consignee |  |
| ASME rating | CLASS 300 | Engineered by | PDO |
| Corrosion allowance | 1 mm | Principal | PDO |
| Revision letter | 01 | Contr. job no. |  |
| Revision date | 01/2024 | Project no. | General |

COMPONENT MATERIAL DESCRIPTION
(For full material description see relevant MESC buying description.)

| Pipe |  |  |  |
| --- | --- | --- | --- |
| PIPE | Pipe | DN 15 | ASTM A106-B |
| PIPE | Pipe | DN 50-600 | ASTM A106-B |

| Flanges |  |  |  |
| --- | --- | --- | --- |
| BLFL | Blind flange | DN 50-600 | ASTM A182-F55 |
| LJFL | Lap joint flange | --- | ASTM A105 |
| SBNF | Spectacle blind | DN 50-350 | ASTM A240 UNS S32760 |
| SPNF | Spade | DN 50-600 | ASTM A240 UNS S32760 |
| SRNF | Spacer ring | DN 50-600 | ASTM A240 UNS S32760 |
| WNFL | Welding neck flange | DN 50-600 | ASTM A105 |

| Fittings |  |  |  |
| --- | --- | --- | --- |
| CAPB | Cap bw | DN 50-600 | ASTM A234-WPB |
| CAPT | Cap thrd | DN 20-40 | ASTM A105 |
| E45B | Elbow 45 deg LR bw | DN 50-600 | ASTM A234-WPB |
| E90B | Elbow 90 deg LR bw | DN 50-600 | ASTM A234-WPB |
| TEEB | Tee equal bw | DN 50-600 | ASTM A234-WPB |

| Reducing fittings |  |  |  |
| --- | --- | --- | --- |
| BN3T | Branch fitting thrd | DN 50-600 | ASTM A105 |
| BN3P | Branch fitiing plain | DN 40-600 | ASTM A105 |
| BR3B | Branch outlet bw | DN 450-500 | ASTM A105 |
| BR6B | Branch outlet bw | DN 600-600 | ASTM A105 |
| BRSB | Branch outlet bw | DN 150-400 | ASTM A105 |
| BROB | Branch outlet bw | DN 400-600 | ASTM A105 |
| RECB | Reducer concentric bw | DN 80-600 | ASTM A234-WPB |
| REEB | Reducer eccentric bw | DN 80-600 | ASTM A234-WPB |
| TERB | Tee reducing bw | DN 80-600 | ASTM A234-WPB |

| Valves |  |  |  |
| --- | --- | --- | --- |
| BARF | Ball valve float RB flgd | DN 50-50 | Body: ASTM A182-F55/A995-6A |
|  |  |  | Trim: Super Duplex, filled PTFE |
| BTFF | Ball valve trunn FB flgd | DN 50-600 | Body: ASTM A182-F55/A995-6A |
|  |  |  | Trim: Super Duplex, filled PTFE |
| BTRF | Ball valve trunn RB flgd | DN 80-600 | Body: ASTM A182-F55/A995-6A |
|  |  |  | Trim: Super Duplex, filled PTFE |
| BUTF | Butterfly valve triple of flgd | DN 200-600 | Body: ASTM A995-6A |
|  |  |  | Trim: Super Duplex, Stellite |
| CHDF | Check valve dual plate flgd | DN 300-600 | Body: ASTM A995-6A |
|  |  |  | Trim: Super Duplex, Stellite |
| CHVF | Check valve flgd | DN 50-600 | Body: ASTM A182-F55/A995-6A |
|  |  |  | Trim: Super Duplex, filled PTFE |
| GAVF | Gate valve flgd | DN 50-600 | Body: ASTM A182-F55/A995-6A |
|  |  |  | Trim: Super Duplex, filled PTFE |
| GLVF | Globe valve flgd | DN 50-200 | Body: ASTM A182-F55/A995-6A |
|  |  |  | Trim: Super Duplex, filled PTFE |
| MSBH | Monofl Slimline SBB G 3/8 TubF | DN 50-50 | Body: ASTM A182 Grade F53/F55 |
|  |  |  | Trim: SDPX stainless/ PTFE |
| VSBH | Monofl ball SBB G 3/8 Tub F | DN 50-50 | Body: ASTM A182 Grade F53/F55 |
|  |  |  | Trim: SDPX stainless/ PTFE |
| MDBH | Monofl Slimline DBB G 3/8 Tub F | DN 50 | Body: ASTM A182 Grade F53/F55 |
|  |  |  | Trim: Super Duplex, filled PTFE |
| VDBH | Monofl ball DBB G 3/8 Tub F | DN 50 | Body: ASTM A182 Grade F53/F55 |
|  |  |  | Trim: Super Duplex, filled PTFE |
| MDRF | Modular DBB Float RB Flanged | DN 50 | Body: ASTM A182 Grade F53/F55 |
|  |  |  | Trim: Super Duplex, filled PTFE |
| MDRF | Modular DBB Float RB Flanged | DN 80 | Body: ASTM A995-6A |
|  |  |  | Trim: Super Duplex, filled PTFE |
| MDFF | Modular DBB Float FB Flanged | DN 50 | Body: ASTM A182 Grade F53/F55 |
|  |  |  | Trim: Super Duplex, filled PTFE |
| MDFF | Modular DBB Float FB Flanged | DN 80 | Body: ASTM A995-6A |
|  |  |  | Trim: Super Duplex, filled PTFE |
| MDOF | Modular DBB Float 10 mm Flanged | DN 50 | Body: ASTM A182 Grade F53/F55 |
|  |  |  | Trim: Super Duplex, filled PTFE |

| Instruments |  |  |  |
| --- | --- | --- | --- |
| TWND | Thermowell flgd DN 50 | DN 50-50 | ASTM A182-F55 |
| TWNS | Thermowell flgd DN 50 L=230 mm | --- | ASTM A182-F55 |
| TWNL | Thermowell flgd DN 50 L=255 mm | --- | ASTM A182-F55 |

| Miscellaneous |  |  |  |
| --- | --- | --- | --- |
| ORFS | Orifice flange set | DN 50-600 | ASTM A182-F55 |
| STBT | Studbolt with nuts | --- | ASTM A193-B7M/A194-2HM |
| GKSW | Gasket spiral wound | DN 50-600 | 6Mo, Graphite |
|  |  |  | CS centring ring/6Mo inner ring |

PIPING COMPONENTS

|  | Nominal pipe size |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 | 20 | 25 | 40 | 50 | 80 | 100 | 150 | 200 | 250 | 300 | 350 | 400 | 450 | 500 | 600 |
| Item description | MESC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Pipe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pipe | 743003 0 | 181 |  |  |  | 691 |  |  |  |  |  |  |  |  |  |  |  |
| Pipe | 743003 1 |  |  |  |  |  | 031 | 491 |  |  |  |  |  |  |  |  |  |
| Pipe | 743003 2 |  |  |  |  |  |  |  | 111 | 411 | 831 |  |  |  |  |  |  |
| Pipe | 743003 3 |  |  |  |  |  |  |  |  |  |  | 251 | 771 |  |  |  |  |
| Pipe | 743003 4 |  |  |  |  |  |  |  |  |  |  |  |  | 251 | 771 |  |  |
| Pipe | 743003 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 291 | 791 |

| Flanges |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blind flange | 766615 |  |  |  |  | 0501 | 0521 | 0541 | 0561 | 0581 | 0601 | 0621 | 0641 | 0661 | 0681 | 0701 | 0721 |
| Spacer ring | 768914 |  |  |  |  | 8001 | 8021 | 8041 | 8061 | 8081 | 8101 | 8121 | 8141 | 8161 | 8181 | 8201 | 8221 |
| Spade | 768952 |  |  |  |  | 0501 | 0521 | 0541 | 0561 | 0581 | 0601 | 0621 | 0641 | 0661 | 0681 | 0701 | 0721 |
| Spectacle blind | 768985 |  |  |  |  | 0501 | 0521 | 0541 | 0561 | 0581 | 0601 | 0621 | 0641 |  |  |  |  |
| Welding neck flange | 766278 |  |  |  |  | 5601 | 5621 | 5641 | 5661 | 5081 | 5101 | 5121 | 5141 | 5161 | 5181 | 5201 | 5241 |

| Fittings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cap bw | 763017 |  |  |  |  | 6051 | 6251 | 6451 | 6651 | 6831 | 7031 | 7231 | 7451 | 7651 | 7861 | 8071 | 8281 |
| Elbow 45 deg LR bw | 763034 |  |  |  |  | 6051 | 6251 | 6451 | 6651 | 6831 | 7031 | 7231 | 7451 | 7651 | 7861 | 8071 | 8281 |
| Cap thrd | 760518 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Elbow 90 deg LR bw | 763042 |  |  |  |  | 6051 | 6251 | 6451 | 6651 | 6831 | 7031 | 7231 | 7451 | 7651 | 7861 | 8071 | 8281 |
| Tee equal bw | 763081 |  |  |  |  | 6051 | 6251 | 6451 | 6651 | 6831 | 7031 | 7231 | 7451 | 7651 | 7861 | 8071 | 8281 |

PIPING COMPONENTS

|  | Nominal pipe size |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 | 20 | 25 | 40 | 50 | 80 | 100 | 150 | 200 | 250 | 300 | 350 | 400 | 450 | 500 | 600 |
| Item description | MESC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Valves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ball valve float RB flgd | 770345 |  |  |  |  | 3581 |  |  |  |  |  |  |  |  |  |  |  |
| Ball valve trunn FB flgd | 770366 |  |  |  |  | 5581 | 5601 | 5621 | 5641 | 5651 | 5661 | 5671 | 5681 | 5691 | 5701 | 5711 | 5731 |
| Ball valve trunn RB flgd | 770366 |  |  |  |  |  | 0601 | 0621 | 0641 | 0651 | 0661 | 0671 | 0681 | 0691 | 0701 | 0711 | 0731 |
| Butterfly valve triple of flgd | 774471 |  |  |  |  |  |  |  |  | 3651 | 3661 | 3671 | 3681 | 3691 | 3701 | 3711 | 3731 |
| Check valve dual plate flgd | 776809 |  |  |  |  |  |  |  |  |  |  | 3671 | 3681 | 3691 | 3701 | 3711 | 3731 |
| Check valve flgd | 771342 |  |  |  |  | 0591 | 3601 | 3621 | 3641 | 3651 | 3661 | 3671 | 3681 | 3691 | 3701 | 3711 | 3731 |
| Gate valve flgd | 772345 |  |  |  |  | 0591 | 6601 | 6621 | 6641 | 6651 | 6661 | 6671 | 6681 | 6691 | 6701 | 6711 | 6731 |
| Globe valve flgd | 773343 |  |  |  |  | 0591 | 6601 | 6621 | 6641 | 6651 |  |  |  |  |  |  |  |
| Monofl ball SBB G 3/8 Tub F | 77PSET |  |  |  |  | B059 |  |  |  |  |  |  |  |  |  |  |  |
| Monofl Slimline SBB G 3/8 TubF | 77NDET |  |  |  |  | B059 |  |  |  |  |  |  |  |  |  |  |  |
| Monofl Slimline DBB G 3/8 Tub F | 77NXET |  |  |  |  | B059 |  |  |  |  |  |  |  |  |  |  |  |
| Monofl ball DBB G 3/8 Tub F | 77QQET |  |  |  |  | B059 |  |  |  |  |  |  |  |  |  |  |  |
| Modular DBB Float RB Flanged | 77RSET |  |  |  |  | B059 | B069 |  |  |  |  |  |  |  |  |  |  |
| Modular DBB Float FB Flanged | 77RGET |  |  |  |  | B059 | B069 |  |  |  |  |  |  |  |  |  |  |
| Modular DBB Float 10 mm Flanged | 77SYET |  |  |  |  | B059 |  |  |  |  |  |  |  |  |  |  |  |

| Miscellaneous |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orifice flange set | 766691 |  |  |  |  | 0811 | 0831 | 0011 | 0021 | 0031 | 0041 | 0051 | 0071 | 0091 | 0211 | 0231 | 0331 |
| Gasket spiral wound | 854158 |  |  |  |  | 1141 | 1181 | 0701 | 0741 | 0761 | 0781 | 0801 | 0821 | 0841 | 0861 | 0881 | 0921 |

REDUCING/PIPING COMPONENTS

| Run | x | Br | RECB | REEB | TERB |
| --- | --- | --- | --- | --- | --- |
| 80 | x | 50 | 7630662351 | 7630672351 | 7630881321 |
| 100 | x | 50 | 7630662651 | 7630672651 | 7630881531 |
| 100 | x | 80 | 7630662751 | 7630672751 | 7630881651 |
| 150 | x | 80 | 7630662951 | 7630672951 | 7630881771 |
| 150 | x | 100 | 7630663051 | 7630673051 | 7630881891 |
| 200 | x | 100 | 7630663241 | 7630673241 | 7630882111 |
| 200 | x | 150 | 7630663341 | 7630673341 | 7630882311 |
| 250 | x | 100 | 7629473421 | 7629623421 | 7630882591 |
| 250 | x | 150 | 7630663521 | 7630673521 | 7630882771 |
| 250 | x | 200 | 7630663631 | 7630673631 | 7630882901 |
| 300 | x | 150 | 7630663811 | 7630673811 | 7630883161 |
| 300 | x | 200 | 7630663941 | 7630673941 | 7630883271 |
| 300 | x | 250 | 7630664031 | 7630674031 | 7630883491 |
| 350 | x | 150 | 7630664121 | 7630674121 |  |
| 350 | x | 200 | 7630664211 | 7630674211 | 7630883891 |
| 350 | x | 250 | 7630664331 | 7630674331 | 7630884141 |
| 350 | x | 300 | 7630664431 | 7630674431 | 7630884401 |
| 400 | x | 200 | 7630664521 | 7630674521 |  |
| 400 | x | 250 | 7630664611 | 7630674611 | 7630885041 |
| 400 | x | 300 | 7630664731 | 7630674731 | 7630885301 |
| 400 | x | 350 | 7630664841 | 7630674841 | 7630885551 |
| 450 | x | 250 | 7630665211 | 7630675211 | 7630886031 |
| 450 | x | 300 | 7630665511 | 7630675511 | 7630886271 |
| 450 | x | 350 | 7630665821 | 7630675821 | 7630886511 |
| 450 | x | 400 | 7630666121 | 7630676121 | 7630886751 |
| 500 | x | 300 | 7630666411 | 7630676411 | 7630887471 |
| 500 | x | 350 | 7630666721 | 7630676721 | 7630887711 |
| 500 | x | 400 | 7630667021 | 7630677021 | 7630887951 |
| 500 | x | 450 | 7630667321 | 7630677321 | 7630888191 |
| 600 | x | 300 |  |  | 7630888671 |
| 600 | x | 350 |  |  | 7630888911 |
| 600 | x | 400 | 7630668821 | 7630678821 | 7630889151 |
| 600 | x | 450 | 7630669121 | 7630679121 | 7630889391 |
| 600 | x | 500 | 7630669421 | 7630679421 | 7630889631 |

| Comp.name | Description |
| --- | --- |
| BROB | Branch outlet bw |
| BR3B | Branch outlet bw |
| BR6B | Branch outlet bw |
| BRSB | Branch outlet bw |
| RECB | Reducer concentric bw |
| REEB | Reducer eccentric bw |
| TERB | Tee reducing bw |
| BN3T | Branch fitting thrd |
| BN3P | Branch fiting plain |

| Thermowell flgd DN 50 |  |
| --- | --- |
| Length (mm) | MESC number |
| 230 | 7683378211 |
| 255 | 7683378221 |
| 305 | 7683378231 |
| 355 | 7683378241 |
| 405 | 7683378251 |
| 455 | 7683378261 |

REDUCING/PIPING COMPONENTS

| Branch | Run |  | pipe | BN3P | BR3B | BR6B | BROB | BRSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 50 | - | 50 | 7680310371 |  |  |  |  |
| 15 | 80 | - | 200 | 7680310381 |  |  |  |  |
| 15 | 250 | - | 500 | 7672692041 |  |  |  |  |
| 50 | 150 | - | 150 |  |  |  |  | 7678312171 |
| 50 | 200 | - | 250 |  |  |  |  | 7678312181 |
| 50 | 300 | - | 400 |  |  |  |  | 7678312201 |
| 50 | 450 | - | 450 |  | 7678311501 |  |  |  |
| 50 | 500 | - | 500 |  | 7678311541 |  |  |  |
| 50 | 600 | - | 600 |  |  | 7678903051 |  |  |
| 80 | 200 | - | 200 |  |  |  |  | 7678312281 |
| 80 | 250 | - | 250 |  |  |  |  | 7678312291 |
| 80 | 300 | - | 350 |  |  |  |  | 7678312301 |
| 80 | 400 | - | 400 |  |  |  |  | 7678312321 |
| 80 | 450 | - | 500 |  | 7678311621 |  |  |  |
| 80 | 600 | - | 600 |  |  | 7679031241 |  |  |
| 100 | 300 | - | 350 |  |  |  |  | 7678312391 |
| 100 | 400 | - | 400 |  |  |  |  | 7678312411 |
| 100 | 450 | - | 500 |  | 7678311711 |  |  |  |
| 100 | 600 | - | 600 |  |  | 7679031351 |  |  |
| 150 | 350 | - | 350 |  |  |  |  | 7678312471 |
| 150 | 400 | - | 400 |  |  |  |  | 7678312481 |
| 150 | 450 | - | 450 |  | 7678311801 |  |  |  |
| 150 | 500 | - | 500 |  | 7678311811 |  |  |  |
| 150 | 600 | - | 600 |  |  |  | 7679004081 |  |
| 200 | 400 | - | 400 |  |  |  | 7678310461 |  |
| 200 | 450 | - | 450 |  |  |  | 7679004661 |  |
| 200 | 500 | - | 500 |  |  |  | 7679004681 |  |
| 200 | 600 | - | 600 |  |  |  | 7679004721 |  |
| 250 | 500 | - | 500 |  |  |  | 7679004841 |  |
| 250 | 600 | - | 600 |  |  |  | 7679004881 |  |

| Branch | Run |  | pipe | BN3T |
| --- | --- | --- | --- | --- |
| 20 | 50 | - | 50 | 7680319861 |
| 25 | 80 | - | 80 | 7680319901 |
| 25 | 100 | - | 100 | 7680319911 |
| 40 | 150 | - | 150 | 7680319961 |
| 40 | 200 | - | 300 | 7680319971 |
| 40 | 350 | - | 600 | 7680319981 |

BOLTING

| Std bolt set flange Joint with PE x PE Lining |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 110 | 8138662741 |
| 80 | 8 | 3/4 | x | 130 | 8138663341 |
| 100 | 8 | 3/4 | x | 140 | 8138663351 |
| 150 | 12 | 3/4 | x | 150 | 8138663371 |
| 200 | 12 | 7/8 | x | 180 | 8138664101 |
| 250 | 16 | 1 | x | 220 | 8138664821 |
| 300 | 20 | 1 1/8 | x | 240 | 8138665431 |
| 350 | 20 | 1 1/8 | x | 250 | 8138665441 |
| 400 | 20 | 1 1/4 | x | 270 | 8138666051 |
| 450 | 20 | 1 1/4 | x | 290 | 8138666081 |
| 500 | 24 | 1 1/ 4 | x | 300 | 8138666101 |
| 600 | 24 | 1 1/2 | x | 350 | 8138667041 |

| Std bolt set flange Joint with PE x Roto Lining |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 110 | 8138662741 |
| 80 | 8 | 3/4 | x | 130 | 8138663341 |
| 100 | 8 | 3/4 | x | 130 | 8138663341 |
| 150 | 12 | 3/4 | x | 140 | 8138663351 |
| 200 | 12 | 7/8 | x | 160 | 8138664071 |
| 250 | 16 | 1 | x | 210 | 8138664801 |
| 300 | 20 | 1 1/8 | x | 230 | 8138665411 |
| 350 | 20 | 1 1/8 | x | 240 | 8138665431 |
| 400 | 20 | 1 1/4 | x | 260 | 8138666031 |
| 450 | 20 | 1 1/4 | x | 270 | 8138666051 |
| 500 | 24 | 1 1/ 4 | x | 280 | 8138666071 |
| 600 | 24 | 1 1/2 | x | 320 | 8138667001 |

| Std bolt set flange Joint with Roto x Roto Lining |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 100 | 8138662731 |
| 80 | 8 | 3/4 | x | 120 | 8138663321 |
| 100 | 8 | 3/4 | x | 130 | 8138663341 |
| 150 | 12 | 3/4 | x | 140 | 8138663351 |
| 200 | 12 | 7/8 | x | 150 | 8138664061 |
| 250 | 16 | 1 | x | 200 | 8138664971 |
| 300 | 20 | 1 1/8 | x | 220 | 8138665401 |
| 350 | 20 | 1 1/8 | x | 220 | 8138665401 |
| 400 | 20 | 1 1/4 | x | 240 | 8138666001 |
| 450 | 20 | 1 1/4 | x | 250 | 8138666091 |
| 500 | 24 | 1 1/ 4 | x | 260 | 8138666031 |
| 600 | 24 | 1 1/2 | x | 290 | 8138666801 |

| Std bolt set flange Joint with PE Lining x Valve |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 100 | 8138662731 |
| 80 | 8 | 3/4 | x | 120 | 8138663321 |
| 100 | 8 | 3/4 | x | 130 | 8138663341 |
| 150 | 12 | 3/4 | x | 140 | 8138663351 |
| 200 | 12 | 7/8 | x | 160 | 8138664071 |
| 250 | 16 | 1 | x | 200 | 8138664971 |
| 300 | 20 | 1 1/8 | x | 220 | 8138665401 |
| 350 | 20 | 1 1/8 | x | 230 | 8138665411 |
| 400 | 20 | 1 1/4 | x | 250 | 8138666091 |
| 450 | 20 | 1 1/4 | x | 260 | 8138666031 |
| 500 | 24 | 1 1/ 4 | x | 270 | 8138666051 |
| 600 | 24 | 1 1/2 | x | 310 | 8138666981 |

| Std bolt set flange Joint with RL x Valve / flange |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 90 | 8138662711 |
| 80 | 8 | 3/4 | x | 110 | 8138663301 |
| 100 | 8 | 3/4 | x | 120 | 8138663321 |
| 150 | 12 | 3/4 | x | 130 | 8138663341 |
| 200 | 12 | 7/8 | x | 150 | 8138664061 |
| 250 | 16 | 1 | x | 190 | 8138664771 |
| 300 | 20 | 1 1/8 | x | 210 | 8138665381 |
| 350 | 20 | 1 1/8 | x | 220 | 8138665401 |
| 400 | 20 | 1 1/4 | x | 230 | 8138665981 |
| 450 | 20 | 1 1/4 | x | 240 | 8138666001 |
| 500 | 24 | 1 1/ 4 | x | 250 | 8138666091 |
| 600 | 24 | 1 1/2 | x | 280 | 8138666931 |

| Std bolt set blind/spacer With PE x PE Lining |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 130 | 8138662781 |
| 80 | 8 | 3/4 | x | 150 | 8138663371 |
| 100 | 8 | 3/4 | x | 160 | 8138663381 |
| 150 | 12 | 3/4 | x | 180 | 8138663411 |
| 200 | 12 | 7/8 | x | 210 | 8138664151 |
| 250 | 16 | 1 | x | 240 | 8138664851 |
| 300 | 20 | 1 1/8 | x | 270 | 8138665501 |
| 350 | 20 | 1 1/8 | x | 280 | 8138665491 |
| 400 | 20 | 1 1/4 | x | 310 | 8138666111 |
| 450 | 20 | 1 1/4 | x | 330 | 8138666141 |
| 500 | 24 | 1 1/ 4 | x | 350 | 8138666171 |
| 600 | 24 | 1 1/2 | x | 400 | 8138667081 |

BOLTING

| Std bolt set blind/spacer PE Lining x Roto |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 120 | 8138662761 |
| 80 | 8 | 3/4 | x | 140 | 8138663351 |
| 100 | 8 | 3/4 | x | 150 | 8138663371 |
| 150 | 12 | 3/4 | x | 170 | 8138663401 |
| 200 | 12 | 7/8 | x | 200 | 8138664131 |
| 250 | 16 | 1 | x | 230 | 8138664831 |
| 300 | 20 | 1 1/8 | x | 270 | 8138665501 |
| 350 | 20 | 1 1/8 | x | 270 | 8138665501 |
| 400 | 20 | 1 1/4 | x | 300 | 8138666101 |
| 450 | 20 | 1 1/4 | x | 310 | 8138666111 |
| 500 | 24 | 1 1/ 4 | x | 330 | 8138666141 |
| 600 | 24 | 1 1/2 | x | 370 | 8138667131 |

| Std bolt set blind/spacer With Roto x Roto Lining |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 120 | 8138662761 |
| 80 | 8 | 3/4 | x | 140 | 8138663351 |
| 100 | 8 | 3/4 | x | 150 | 8138663371 |
| 150 | 12 | 3/4 | x | 160 | 8138663381 |
| 200 | 12 | 7/8 | x | 180 | 8138664101 |
| 250 | 16 | 1 | x | 230 | 8138664831 |
| 300 | 20 | 1 1/8 | x | 250 | 8138665441 |
| 350 | 20 | 1 1/8 | x | 250 | 8138665441 |
| 400 | 20 | 1 1/4 | x | 280 | 8138666071 |
| 450 | 20 | 1 1/4 | x | 290 | 8138666081 |
| 500 | 24 | 1 1/ 4 | x | 300 | 8138666101 |
| 600 | 24 | 1 1/2 | x | 340 | 8138667021 |

| Std bolt set blind/spacer With PE Lining x Valve |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 120 | 8138662761 |
| 80 | 8 | 3/4 | x | 140 | 8138663351 |
| 100 | 8 | 3/4 | x | 150 | 8138663371 |
| 150 | 12 | 3/4 | x | 170 | 8138663401 |
| 200 | 12 | 7/8 | x | 190 | 8138664121 |
| 250 | 16 | 1 | x | 230 | 8138664831 |
| 300 | 20 | 1 1/8 | x | 250 | 8138665441 |
| 350 | 20 | 1 1/8 | x | 270 | 8138665501 |
| 400 | 20 | 1 1/4 | x | 290 | 8138666081 |
| 450 | 20 | 1 1/4 | x | 300 | 8138666101 |
| 500 | 24 | 1 1/ 4 | x | 320 | 8138666121 |
| 600 | 24 | 1 1/2 | x | 360 | 8138667121 |

| Std bolt set blind/spacer With Roto Lining x Valve |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DN | nr | inch | x | mm | MESC number |
| 50 | 8 | 5/8 | x | 110 | 8138662741 |
| 80 | 8 | 3/4 | x | 130 | 8138663341 |
| 100 | 8 | 3/4 | x | 140 | 8138663351 |
| 150 | 12 | 3/4 | x | 160 | 8138663381 |
| 200 | 12 | 7/8 | x | 180 | 8138664101 |
| 250 | 16 | 1 | x | 220 | 8138664821 |
| 300 | 20 | 1 1/8 | x | 240 | 8138665431 |
| 350 | 20 | 1 1/8 | x | 250 | 8138665441 |
| 400 | 20 | 1 1/4 | x | 270 | 8138666051 |
| 450 | 20 | 1 1/4 | x | 280 | 8138666071 |
| 500 | 24 | 1 1/ 4 | x | 300 | 8138666101 |
| 600 | 24 | 1 1/2 | x | 330 | 8138667011 |

ASSEMBLIES

| Drain or vent point DN 50 Fig.H DN 50 - 600 |  |  |
| --- | --- | --- |
| Branch: according to branch table |  |  |
| Blind flange | 1 | 7666150501 |
| Welding neck flange | 1 | 7662785601 |
| Studbolt with nuts | 8 | 8138662761 |
| Gasket spiral wound | 1 | 8541581141 |

| PI.conn.monofl.sliml DN 50 Fig.PX1 DN50 - 100 |  |  |
| --- | --- | --- |
| Branch: according to branch table |  |  |
| Welding neck flange | 1 | 7662785601 |
| Monofl Slimline SBB G 3/8 TubF | 1 | 77NDETB059 |
| Studbolt with nuts | 8 | 8138662761 |
| Gasket spiral wound | 1 | 8541581141 |

| PI.conn.monofl.sliml DN 50 Fig.PX2 DN150 - 600 |  |  |
| --- | --- | --- |
| Branch: according to branch table |  |  |
| Welding neck flange | 1 | 7662785601 |
| Monofl Slimline SBB G 3/8 TubF | 1 | 77NDETB059 |
| Studbolt with nuts | 8 | 8138662761 |
| Gasket spiral wound | 1 | 8541581141 |

| Temp instr conn DN 50 Fig.TX-1 DN80- 100 |  |  |
| --- | --- | --- |
| Branch: branch nipple flanged |  |  |
| Lap joint flange | 1 | 7662202181 |
| Thermowell flgd DN 50 L=230 mm | 1 | 7683378211 |
| Studbolt with nuts | 8 | 8138662761 |
| Pipe spool (50mm long) | 1 | 7402030691 |
| Welding neck flange | 1 | 7662785601 |
| Gasket spiral wound | 1 | 8541581141 |

| Temp instr conn DN 50 Fig.TX-2 DN150- 600 |  |  |
| --- | --- | --- |
| Branch: branch nipple flanged |  |  |
| Lap joint flange | 1 | 7662202181 |
| Thermowell flgd DN 50 L=255 mm | 1 | 7683378221 |
| Studbolt with nuts | 8 | 8138662761 |
| Welding neck flange | 1 | 7662785601 |
| Gasket spiral wound | 1 | 8541581141 |

ALTERNATIVE ASSEMBLY

Sketch 1

| PI.conn.monofl.ball. DN 50 Fig.AA DN50 – 600 |  |  |
| --- | --- | --- |
| Branch: according to branch table |  |  |
| Welding neck flange | 1 | 7662785601 |
| Monofl ball SBB Flanged | 1 | 77PSETB059 |
| Studbolt with nuts | 8 | 8138662761 |
| Gasket spiral wound | 1 | 8541581141 |

| Annulus vent point DN 15 DN50 – 50 (Sketch-1) |  |  |
| --- | --- | --- |
| Branch: Branch fitting plain |  |  |
| 1/2” pipe 0.5 m length | 1 | 7430030181 |
| Branch fitting plain | 1 | 7680310371 |

| Annulus vent point DN 15 DN80 – 200 (Sketch-1) |  |  |
| --- | --- | --- |
| Branch: Branch fitting plain |  |  |
| 1/2” pipe 0.5 m length | 1 | 7430030181 |
| Branch fitting plain | 1 | 7680310381 |

| Annulus vent point DN 15 DN250 – 600 (Sketch-1) |  |  |
| --- | --- | --- |
| Branch: Branch fitting plain |  |  |
| 1/2” pipe 0.5 m length | 1 | 7430030181 |
| Branch fitting plain | 1 | 7672692041 |

| Pipe size | PE Lining (mm) | PE Lining (mm) | Roto lining (mm) |
| --- | --- | --- | --- |
| DN | Pipe | Flange stub end | Pipe / Flange stub end |
| 50 | 6.00 | 12.00 | 5.00 |
| 80 | 6.00 | 12.00 | 5.00 |
| 100 | 6.00 | 12.00 | 6.00 |
| 150 | 6.00 | 12.00 | 7.00 |
| 200 | 7.00 | 14.00 | 7.00 |
| 250 | 8.00 | 16.00 | 8.00 |
| 300 | 10.00 | 20.00 | 8.00 |
| 350 | 11.00 | 22.00 | 8.00 |
| 400 | 13.00 | 26.00 | 8.00 |
| 450 | 14.00 | 28.00 | 8.00 |
| 500 | 16.00 | 32.00 | 8.00 |
| 600 | 19.00 | 38.00 | 8.00 |